A Primer on IP Addressing, Subnetting, and Netmasks

---------------------------------------------------

With the Internet Connectivity Option version 2.0, you can assign specific IP

addresses to specific users so they can connect to your server with a

SLIP, CSLIP, or PPP connection and run their own servers on the Internet.  You

can also support dynamic IP addresses for occasional users.

None of this is necessary (and you don't need to bother with subnetting) if

all your users want to have SLIP, CSLIP or PPP connections to run Netscape or

other Internet-browsing packages.  It *is* necessary to create subnets if any

of your users need to put their own server computers on the Internet, or if

anyone needs to run certain packages, such as the RealAudio Player, or an FTP

client that only uses the PORT command (instead of the PASV command like

Netscape uses).

We'll start with some theory and then go through a specific example.

IP Addressing

-------------

TCP/IP addressing has some flexible ways of identifying networks, identifying

hosts within a network, and creating new networks.

IP addresses are composed of 32 bits or 4 bytes.  For example:

199.227.15.14

Each of these four numbers (or bytes) can range from 0 to 255.

IP addresses are also composed of two parts:

Network part   Identifies a network

Host part      Identifies a host computer within a network

Some bits are reserved for the network part, the rest are for the host

part.  The standard partition between network bits and host bits works

differently for different ranges of IP addresses, called classes.  The three

most important classes are A, B, and C.

Approximate

number of

Number of     hosts within

Class        IP address range            networks     each network

-----  ----------------------------      --------     ------------

A      0.0.0.0 to 127.255.255.255           128       16 million

B    128.0.0.0 to 191.255.255.255        16,384      64 thousand

C    192.0.0.0 to 223.255.255.255     2,097,152              253

Almost all new networks going online today are using class C addresses.

Class C addresses can support about 2 million networks:

(223-192+1) x 256 x 256   =   2,097,152 class C networks

Each class C network can nominally support up to 256 host computers.  Reality

falls slightly short of nominal in that some addresses are reserved.  That is:

199.227.15.0        - represents the entire network

199.227.15.254      - represents the local IP address of the

Worldgroup server on a SLIP/CSLIP/PPP

pseudo-network (no such address is

used up on an Ethernet however)

199.227.15.255      - is the destination address for packets to

be broadcast to all hosts on the network

These first and last addresses would normally never be used to designate host

computers.  The .0 address is used in certain contexts, mostly router

configuration, to designate the entire network.  And the .255 address is used

by a broadcast packet to address all hosts within a network.  The way IP

addresses are used for SLIP/CSLIP/PPP users, the Worldgroup server needs an

address on the network too.  It always uses the one right before the broadcast

address.

Here's what's left over on a typical set of class C addresses that are

allocated to SLIP/CSLIP/PPP users:

199.227.15.1 through 199.227.15.253     - addresses for SLIP/CSLIP/PPP

users on the pseudo-network

So this is how a class C network or set of addresses is broken down:  the

string of 256 addresses, only varying in the fourth number, includes at least

a few addresses for special purposes.  But intrepid TCP/IP network

administrators, such as yourself, can also carve smaller networks out of the

standard networks.  These are subnetworks or subnets.

Subnetting

----------

To assign IP addresses to SLIP/CSLIP/PPP users, you need to create

a network, or sub-network, of IP addresses.  These users are, at least

conceptually, all part of the SLIP/CSLIP/PPP subnetwork.

Breaking down a standard-sized network into smaller networks is almost always

done with class A or class B networks, but it can also be done with class C

networks.  You can cut a class C network in half and have two 128-address

networks.  Or you could cut it into one 128-address network and four

32-address networks.  (All numbers are nominal of course.)  Netmasks show

where the split takes place.

The (nominal) number of hosts in a network is always a power of two, so that

the 32 bits of the IP address can be split cleanly into a network part and a

host part.  This keeps the job of the network routers dirt-simple, so they can

route lots of packets very quickly.

So the only earthly purpose for netmasks is to empower networking software to

make these cuts:  to separate the part of the IP address that specifies the

network, from the part that specifies a host within that network.

Netmasks

--------

A netmask looks a lot like an IP address but it's really something quite

different.  Here's the standard netmask for Class C addresses:

255.255.255.0

This netmask means that the left 24 bits (the left 3 bytes, as designated by

the 255's) of an IP address are the network part and the right 8 bits (byte)

is the host part.  To create class C subnets, you have to split up those 8

bits on the right.  Here are the possibilities.

Rightmost byte                           Practical number

of the netmask                           of hosts within

a SLIP/CSLIP/PPP

Binary    Decimal    Number of subnets    pseudo-subnetwork

--------  -------    -----------------    -----------------

11111111    255            256                    0 (impractical)

11111110    254            128                    0 (impractical)

11111100    252             64                    1 (not very practical)

11111000    248             32                    5

11110000    240             16                   13

11100000    224              8                   29

11000000    192              4                   61

10000000    128              2                  125

00000000      0              1                  253 (no subnetting)

A netmask of 255.255.255.192, therefore, can be used to break a class C

network into 4 subnets with up to 61 computers on each subnet.  (Remember,

three address values are reserved within each SLIP/CSLIP/PPP subnetwork, but

only two on an Ethernet subnetwork).

By the way, below are the standard netmasks for any of you who are lucky

enough to be working with class A or B IP addresses.

Standard

Class   IP address of your server    Netmask

-----  ----------------------------  -------

A      0.0.0.0 to 127.255.255.255  255.0.0.0

B    128.0.0.0 to 191.255.255.255  255.255.0.0

C    192.0.0.0 to 223.255.255.255  255.255.255.0

If your subnets are all smaller than 256 addresses (and most likely that's

what you want), you'll want a netmask starting with 255.255.255.  For the

rightmost byte of the netmask, use one of the decimal numbers in the above

table.

What your Service Provider says is your Netmask

-----------------------------------------------

Your service provider may tell you what "your" netmask is.  This isn't

necessarily the netmask you'll use in all cases however.  It's really just

telling you how many addresses your ISP is giving you to play with.  (See

the above table.)  If you had only one network, then you would probably use

that same netmask in all contexts.

But if you're offering SLIP/CSLIP/PPP services to your users, then you have at

least one additional network -- the SLIP/CSLIP/PPP pseudo-network -- and you

need to compute your own netmasks for the purpose.

Router Configuration

--------------------

There's a special advantage to splitting up a class C network into one part

for Ethernet connections and one for SLIP/CSLIP/PPP connections through the

Worldgroup server:  the router doesn't need to know about it.  The router can

pretend that all 254 possible addresses of the class C set are on the local

LAN.  So it can use the standard netmask for class C networks:  255.255.255.0.

The Worldgroup server, on the other hand, through strategic setting of its

netmasks and addresses, can take care of routing the packets destined for

SLIP/CSLIP/PPP users.  (By the way, the technical feature that makes this

possible is that ICO will respond to ARP requests on the LAN for the

SLIP/CSLIP/PPP users by claiming to be the proper recipient for those packets

on the LAN.)  This cutting in two of a class C set of addresses is exactly

what we'll see in the following example.

Example

-------

Suppose you have a T1 connection to your Internet Service Provider, and he's

also allocated you your own class C network.  Let's say that 199.221.88.0

through 199.221.88.255 are yours to play with.  (The Service Provider would

probably tell you in this case that your IP addresses start at 199.221.88.1

and your netmask is 255.255.255.0.  Don't let that fool you into using that

netmask in all contexts though.  Read on.)

We'll chop the IP addresses cleanly in two so that half of the addresses are

for hosts and devices on the Ethernet connected to the T1 line, and half are

for assignment or dynamic allocation to users.

T1

||

____||____

|          |

|   DSU    |

|____  ____|

____||____                       _________

|          |                     |         |

|  Router  |                     |  Host   |

|__________|                     |_________|

|                               |

| 199.221.88.1                  | 199.221.88.4

|                               |

_________________|_______________________________|___________________________

TCP/IP Ethernet LAN          |            |             |

|            |             |

|            |             |

199.221.88.2 |            |             | 199.221.88.5

____|____        |         ____|____

|         |       |        |         |

|  Host   |       |        |  Host   |

|_________|       |        |_________|

|

|

| 199.221.88.3

________|________

|                 |

|   Worldgroup    |

|     Server      |

|_________________|

|             |

|             |

199.221.88.129  |             | 199.221.88.130

________|___        __|__________

|            |      |             |

|  PPP user  |      |  SLIP user  |

|____________|      |_____________|

In this example, the PPP user has been assigned his own IP address,

199.221.88.129.  The SLIP user is using 199.221.88.130.

Here are some reasonable configuration settings for this arrangement:

GALTCPIP.MSG

------------

IPADDR    IP address of Worldgroup server (or AUTO): .... 199.221.8.3

NETMASK   Netmask: .................................. 255.255.255.128

IPROUT    IP address of router: ......................... 199.221.8.1

GALSCP.MSG

----------

SCPMAX    Maximum number of SLIP/CSLIP/PPP sessions:              125

USERIPS   Support assigned and dynamic user IP addresses?         YES

SCPIP     SLIP/CSLIP/PPP sample assigned IP address:   199.221.88.129

SCPMASK   SLIP/CSLIP/PPP netmask (or AUTO):           255.255.255.128

Notice that the netmasks end in 128, which is 10000000 in binary.  (Recall

from the above table, that this designates subnets of up to 125 hosts each.)

This 10000000 binary means that, within the set of class C addresses starting

with 199.221.88, bit 7 (numbered from the right starting at 0) determines

whether a packet is associated with a SLIP/CSLIP/PPP user or with a host on

the local Ethernet.  Bits 0-6 determine which user or host within those groups

a packet is for.  Router algorithms love to deal in bits.

Another way of saying this:

Netmask 255.255.255.128 allows:

-------------------------------

199.221.88.1   - 199.221.88.126  to be used for Ethernet hosts

199.221.88.129 - 199.221.88.253  to be assigned to SLIP/CSLIP/PPP users

Remember that every subnet must contain an integral power of two addresses

(16, 32, 64, or 128 for example).

With the standard netmask 255.255.255.0, all addresses starting with

199.221.88 must be associated with a single network -- there is no subnetting.

The special netmask 255.255.255.128, which both NETMASK and SCPMASK are set

to, splits the class C set of addresses into two subnetworks.

Even though there are exactly 125 available IP address for SLIP/CSLIP/PPP

users in this example, you wouldn't necessarily set SCPMAX to 125.  You might

set it to a lower value to limit Internet traffic for some reason (the nominal

T1 rate is about 40 times that of a 38400 bps connection for example).  You

might set SCPMAX to a higher value because of all the proxy SLIP/CSLIP/PPP

users you want to accommodate (who don't need their own IP addresses).

The SCPIP setting, in this case 199.221.88.129, doesn't have to be the

starting address.  As long as you specify any address in the range you want

for SLIP/CSLIP/PPP users, the first and last addresses are determined by the

netmask.